- BT PTT version ET06_PTT_V15

- 1.Resolved the issue that BT PTT battery voltage show on D878UV PLUS is incorrect.

- 2. Resolved the issue that BT PTT disconnect once D878UV PLUS is reboot.

- How to Check the BT PTT version

- 1.Install elet-tool1.2_AN9.0.apk on Android mobilephone

- 2.Run the ElET software, turn on the BT function in mobilephone.

- 3.Click on BLE, at the same time push BT PTT button, the blue light is blinking.

- 4.Select “ELETPTT00AC15”

- 5.Click on “BLE版本查询”, it will show the version ET06_PTT_V15

- How to upgrade BT PTT software

- Install OTA _New tool_V3.0.apk on Android mobilephone

- Run the OTA software

Note: The BT function in mobilephone should be turned on first.

- Click on the “SCANBLE” on the right upper corner.

- Click on “scanbr/edr”, it will start sourcing the device.

- Push BT PTT button, the blue light is blinking.

- The sourced device will be display on mobilephone.

- Click on ELET-PTTXXXXXX, mobilephone will connect to the BT PTT

- Click on “升级bluetooth本地”, it will remind you to select the upgrade file.

- Choose the eeprom_OTA_ET06_PTT_V15.bin file, click on “Upgrade”, it will start the firmware upgrade.

- After the upgrade is done, click on “DISCONNECT”, the BT PTT firmware upgrade is finished.